INHIKEPJ^INE INSPECTION CHECKLIST

FOR ROTAX 2-STROKE AIRCRAFT ENGINES

Fuel tank, quantify and quality Is there enough fuel for the planned flight? What was the octane rating at the

1 pump, is the fuel too old? Does the fuel contain too much alcohol? Is the fuel tank cap back on and secured? Is the
fuel tank vent clear?

2-8troke Engine Oil Is the right type of oil being used (regardless of brand}'?

2 Was oil added to the fuel, usingthe right ratio? (for engines withoutoi! injection)
Is there enough oil in the injection tank? (engines with oil injection)

3 CorrtamlnantB Is there water, dirt or insects inside the fuel tank?

4 Water separation system Has the fuel tank sump and / or gascolator been drained? Are the drain plugs secured? :

5 Fuel filter Is it the right type, is it clogged, cracked and installed correctly?                                                                              '

6 Fuel pump Is it the right type, is it leaking, is it installed correctly and securely?

i Primer pump Plunger type- is it leaking is it worn (can enough resistance be felt)?
Primer bulb if present, is it installed with a bypass circuit, is it cracked or leaking?

o Fuel line Is it the righttype, does it show discoloration, cracks, leaks, stiffness orwear? Is it correctly secured and :

clamped, especially on the fuel pump fittings?

9 Primer line Is it the righttype, does it show discoloration, cracks, leaks, stiffness orwear?

<n Pulse line Is it the righttype, does it show discoloration, cracks, leaks orwear? Is it still flexible? Is it collapsible
between yourfingers?

ENGINE MOUNTING

11 En^rwrnountArethepartss h owi ng anycracks^orrosionordamage?^

12 Rubber mounts Are they showing any signs of cracks, drying deformation or wear?

13 Attaching hardware Are all the nuts, washers and bolts present, tight and not corroded?

ENGINE CONTROLS________________________________________________

14 Control levers Are the levere in good condition, mounted correctly, movingfreelv overthe full range of travel and      ,
stopping adequately?                                                                                                                                                                      ;

15 Control cables Are irfthey good condition and routed correctly? Do they have the right amount of play at the carbu-,
retors?

ENGINE INSTRUMENTS_____________________________________________

16 Hour meter Is one installed, is it operating correctly?

17 Tachometer Is one installed, is itoperating correctly?

18'Dual Cylinder Head and/or liquid temperature gauge is'one installed., is it operating correctly?

19 Dual Exhaust Gas TemperatLire 0iUg9 is one installed, is it operating correctly?"

20 Cylinder Head and/or liquid temperature probes Are they installed and secured correctly? Are'they damaged?

21 'Exhau»tGa«TempetaitiJre probes Are they installed and secured correctly? Are they damaged?' "

AIR FILTER and INTAKE SILENCER (ifliistalliHl)____________________________________

22AirfBterlsitthecorrecttype^sitcleah^                                                                                                              gel or
moist with fuel or water? Is the cover still on?

23 Intake Silencer Is it showing any damage, is it secured correctly?

24 Intake Silencer sockets Are they showing any cracks or damage, are the clamps tight?

CARBURETORS___________________________________________

25 Angle Are they correctly angled out of the engine and at a right angle to the crankcase?
28 Rubber caps Are they present, are they cracked?

27 Vent lines Do they have the right length, right size holes, are they in good condition, any obstructions?

28 Rubber sockets Are they showing any cracks, are they correctly tightened?

29 Float bowls Are the clips too easy to undo? Is there any water, varnish, dirt orcorrosion?

30 Floatlewl Are the floats floating at the correct level?'Is the float bracket correctly adjusted?

Aero Propulsion

rccHHamelea '

Copyright © 2005